## Interface Control Document CDM CCD Controller

| Author | Institution |  | To be approved by | Institution |
| --- | --- | --- | --- | --- |

| List of Abbreviations |  |  |  |
| --- | --- | --- | --- |
| ACS | Alma Common Software | N/A | Non applicable |
| ACTL | Array Control | OPC-UA |  |
| DP | Data Point | TBC | To be confirmed |
| ICD | Interface Control Document | TBD | To be done |
| LST | Large-Sized Telescope | NUC | Next Unit of Computing |
| CDM | Camera Displacement Monitor |  |  |
| SG | Starguider |  |  |

| Reference documents |  |  |
| --- | --- | --- |
| R1 | MAN-PO/121010a | Systems Engineering Plan |
| R2 | TBD | ICD INFRA-Telescopes (TBD) |

| Applicable documents |  |  |
| --- | --- | --- |
| A1 | ACTL-PM/140115 | Device control integration into the CTA array control and operations |
| A2 | TBD | Ethernet connection parameters for telescopes. (TBD) |
| A3 | TBD | OPCUA and logical device/subsystems namespace. (TBD) |

| History |  |  |
| --- | --- | --- |
| Version | Date | Observation |
| 1.0 | 22/08/2018 | First ICD for CDM CCD |
| 2.0 | 18/09/2019 | Real ICD From CDM CCD (reverse engineering !!!!!!) |
| 2.1 | 15/11/2019 | Add ImagePath datapoint |
| 2.2 | 12/12/2019 | Add setStarName method |
| 2.3 | 26/03/2020 | Modification with compatibility with new version of codegeneration. Allow to have same name for (method or datapoint in differents folders). |
| 2.4 | 08/04/2020 | Verification avec codegenerateur GIT/CI |
| 2.5 | 30/04/2020 | Avec RemotePath RemoteServer. Mise en place de tableaux + chapitre subServer et subDP pour faire des reference. |
| 3.0 | 01/06/2020 | Refactored. Merged CDM, SG, sensor into one ICD |
| 3.1 | 05/06/2020 | Correction of some mistakes in the document see Red color |
| 3.3 | 08/05/2020 | Branch with only CDM part in this ICD. |
| 4.0 | 02/07/2020 | Removed cdm prefixes. Simplified use cases. Implemented FSM. Configuration datapoints now adjusted via Configure method. |
| 4.1. | 11/09/2020 | Changes some datapoint types (Pixelclock – int32, FPS – double, exposure – double, gain -int32). Changed gain min and max values. Removed number of images to integrate from Configure. Added GetMultipleImages async method. Added nImagesGet datapoint. ImagePath is now an array. |
| 4.2. | 14/09/2020 | Comment function renamed to AddComment. StartCDM now Async. |
| 4.3 | 21/09/2020 | Updated FSM. Added Tpoint and Imaging state. Added GoToTpoint, GoToReady, GetMultipleImages and StopGetMultipleImages. AddComment not a part of FSM. |
| 4.4 | 21/09/2020 | Add State and Transition datapoint for FSM (need for ACS part) |
| 4.5 | 25/09/2020 | Bug fixed (found by Darko) 2 tables merge in one table (temperature and humidity). 2 tag Max_range in GetMultipleImages method |
| 4.6 | 07/10/2020 | Removed output parameters for all methods. Removed input parameter for GetMultipleImages method. |
| 4.7 | 13/10/2020 | Input argument for GetMultipleImages For the AuxController Opcua : Change the ref, and server for |
| 4.8 | 18/10/2020 | Configure argument exposure max range now 1000. Fixed RemoteMethodName of GetImage and StopGetMultipleImages in ICD. Added GetMultipleImagesStacked method. |
| 5.0. | 19/01/2021 | Changed the LEDs to Compund datapoint. Incresead the number of LEDs to 12. Split LED, OARL into x and y coo. datapoints, made them as CompoundDatapoints. Circle, Displacement now compund datapoint. Changed circleErr to circle_sttddev datapoints. Timestamp now compound datapoint with epoch and UTC array subdatapoints. Changed circle values from float to double. Split humidity in absolute and relative humidity. Camera temperature value and camera temperature status datapoint added. Added Stream FSM. Added OARL_mean datapoints. Added Camera Compound datapoint consisting out of FPS, exposure, gain, pixel clock, pixelFormat, temperatureStatus, temperatureValue. Changed default values, mostly to 0. Configure method now callable from Tpoint state, Configure FSM change no longer automatic. FSM now in AuxControl, not in AuxControl.CDM (should it be like this?). |
| 5.1. | 25/02/2021 | Added correct FSM diagram in which the Configure method is callable from Tpoint state. |

| Distribution | CTA internal |
| --- | --- |

Table of Contents

1	CDM CCD subsystem description	4

2	Applicable Use-Cases	5

3	Data exchange	6

3.1	Overview	6

3.2	Hierarchical description of the server for code generation	7

3.3	General description for the code generator	9

3.4	Description of Monitoring Datapoints	10

1.1.1	FSM datapoints (State / Transition)	26

3.5	Description of Configuration Datapoints	29

3.6	Description of configuration Datapoints	36

3.7	Description of sub servers	37

3.8	Description of sub elements	38

3.9	Description of Methods	40

List of Tables:

Table 1 CDM CCD: Applicable Use-Cases.	5

Table 2: Overview	6

Table 3 Hierarchy	8

Table 3 Hierarchy	9

Table 4: General description for code generator	9

Table 5: swVersion	10

Table 68 FSM.state datapoint	26

Table 69 FSM.transition datapoint	27

Table 6 SubServer reference	37

Table 7: SubDP reference	38

List of figures:

Figure 1: CDM FSM	4

# CDM CCD subsystem description

The scope of the document is to describe the software interface of the CDM CCD system which is developed for the LST cameras of the CTA experiment.

Figure 1: CDM FSM

# Applicable Use-Cases

The table below provides a list of use cases and sequence diagrams who allow to define the interface of the CDM CCD sub-system.

| Applicable Use-Cases |  |  |
| --- | --- | --- |
| Code | Name | Description |
| UC-CDM-1 | connect | Connect to the CDM camera |
| UC-CDM-2 | disconnect | Disconnect from the CDM camera |
| UC-CDM-3 | configure | Configure camera settings |
| UC-CDM-4 | getImage | Take and save one CDM image |
| UC-CDM-5 | startCDM | Start continuously taking and analysing CDM images |
| UC-CDM-6 | stopCDM | Stop continuously taking CDM images |
| UC-CDM-7 | error | In case of error occurring go to Undefined state |
| UC-CDM-8 | acknowledge | Go from Undefined state to Safe |
| UC-CDM-9 | goToTpoint | Go to Tpoint state |
| UC-CDM-10 | goToReady | Return to ready state from Tpoint state |
| UC-CDM-11 | getMultipleImages | Take and save multiple CDM images for Tpoint |
| UC-CDM-12 | stopGetMultipleImages | Stop taking Tpoint images |
| UC-CDM-13 | getMultipleImagesStacked | Take multiple CDM images for Tpoints, average them and save as one image |
| UC-CDM-14 | StartStream | Display video feed from the camera |
| UC-CDM-15 | StopStream | Stops the video feed from the camera |

Table 1 CDM CCD: Applicable Use-Cases.

# Data exchange

### Overview

Table 2: Overview

| I-LST-000x: |  |
| --- | --- |
| Monitoring Data Points |  |
| Name | Description |
| Camera / temperatureValue | Temperature value of the camera |
| Camera / temperatureStatus | Temperature status of the camera (normal, warning, critical) |
| Circle / circle_R | Radius of the fitted circle (vector) |
| Circle / circle_x | X coo. of the fitted circle (vector) |
| Circle / circle_y | Y coo. ff the fitted circle (vector) |
| Circle / circle_RMS | Error the fitted circle (vector) |
| Circle / circle_R_stddev | Stddev of the vector of R |
| Circle / circle_x_stddev | Stddev of the vector of x |
| Circle / circle_y_stddev | Stddev of the vector of y |
| Circle / circle_RMS_stddev | Stddev of the vector of RMS |
| Displacement / displacement_x | Circle center – OARL displacement horizontal direction |
| Displacement / displacement_y | Circle center – OARL displacement vertical direction |
| Displacement / rotation | Rotation of the circle |
| LED_x / LED_x_... | Horizontal coo. Of LEDs (multiplicity = 12) |
| LED_y / LED_y_... | Vertical coo. Of LEDs (multiplicity = 12) |
| OARL_x / OARL_x_... | Horizontal coo. Of OARLs (multiplicity = 2) |
| OARL_x / OARL_x_mean | Mean horizontal coo. Of OARLs |
| OARL_y / OARL_y_... | Vertical coo. Of OARLs (multiplicity = 2) |
| OARL_y / OARL_y_mean | Mean vertical coo. Of OARLs |
| Image | Containing the image (bytestring) |
| ImageName | Contains the image name |
| ImagePath | Contains the path of images (vector) |
| ImagePath_cat | Contains contacenated path of images |
| LibVersion | Camera library version |
| Status | CDM status description |
| SwVersion | Version of CDM software |
| timestamp/timestamp_UTC | Timestamp in UTC time |
| timestamp/timestamp_epoch | Timestamp in Unix time |
| Sensor / humidity_absolute | Absolute humidity measured by the sensor |
| Sensor / humidity_relative | Relative humidity measured by the sensor |
| Sensor / pressure | Pressure in the hood measured by the sensor |
| Sensor / temperature | Temperature in the hood measured by the sensor |
| Configuration Data Points |  |
| Camera / FPS | Camera - frames per second setting |
| Camera / exposure | Camera – exposure setting |
| Camera / gain | Camera – gain setting |
| Camera / pixelClock | Camera – pixel clock setting |
| Camera / pixelFormat | Camera – pixel format setting (8bit, 16bit, etc.) |
| Comment | Comment to be added to the image |
| NumIntegrateImage | Number of images to integrate in stacking mode |
| nImagesGet | Number of taken images with GetMultipleImages |
| Methods |  |
| Name | Description |
| Connect | Connect to the CDM camera via USB |
| Disconnect | Disconnect CDM camera from the PC |
| GetImage | Takes a single image image. The output is always one image. |
| Configure | Configures the camera settings. |
| AddComment | This will be added to the image header |
| StartCDM | Start continuously taking and analysing images. |
| StopCDM | Stop continuously taking images. |
| Error | Goes to Undefined state in case of issues. |
| Acknowledge | Goes from Undefined to Safe state. |
| GoToTpoint | Go to Tpoint state |
| GoToReady | Return to ready state from Tpoint state |
| GetMultipleImages | Take and save multiple CDM images |
| StopGetMultipleImages | Stop taking Tpoint images |
| GetMultipleImagesStacked | Take multiple CDM images, average them and save as one image |
| StartStream | Start camera video stream |
| StopStream | Stops camera video stream |

### Hierarchical description of the server for code generation

In order to generate the OPC UA server, you can choose to define a hierarchical structure of the OPCUA server. This hierarchical structure allow to define the unique ID of each datapoints.

| Name_of_element | AuxControl |  |  |
| --- | --- | --- | --- |
| Is_Composed_of | Description | Multiplicity | Start_Number |
| CDM | Camera Displacement monitor | 1 | 1 |
| Sensor | Temperature, humidity, pressure sensor located in the hood | 1 | 1 |
| FSM | Finite state machine | 1 | 1 |

Table 3 Aux control hierarchy

| Name_of_element | AuxControl.CDM |  |  |
| --- | --- | --- | --- |
| Is_Composed_of | Description | Multiplicity | Start_Number |
| Camera | Camera setting and values | 1 | 1 |
| Circle | LED circle results | 1 | 1 |
| Displacement | Displacement between circle center and OARL center | 1 | 1 |
| LED_x | LED horizontal coo. | 1 | 1 |
| LED_y | LED vertical coo. | 1 | 1 |
| OARL_x | OARL horizonal coo. | 1 | 1 |
| OARL_y | OARL vertical coo. | 1 | 1 |
| timestamp | Timestamp of the image | 1 | 1 |

Table 4 CDM hierarhy

### General description for the code generator

Table 5: General description for code generator

| OPC UA server |  |
| --- | --- |
| Description | CDM - LAPP opc-ua interface |
| OPC_UA_address | opc.tcp://auxserver:48011 |
| ACS Bridge |  |
| Prefix | LST |
| ModuleName | Unit_CDM |
| ComponentName | AuxControl |
| ContainerName | AuxiliaryMasterContainer |
| RootPath | Auxiliary |
| MOS_typo | 1 |
| Enable_Remote_Method | 1 |

### Description of Monitoring Datapoints

| Name_of_DataPoint | FPS |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Camera | Group or device name define in the see Hierarchical description of the server |
| Description | CDM frames per second |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | Hz |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | exposure |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Camera | Group or device name define in the see Hierarchical description of the server |
| Description | CDM sensor exposure time |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | ms |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | gain |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Camera | Group or device name define in the see Hierarchical description of the server |
| Description | CDM camera gain |  |
| Data_Type | int32 | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | pixelClock |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Camera | Group or device name define in the see Hierarchical description of the server |
| Description | CDM sensor refresh rate |  |
| Data_Type | int32 | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | pixelFormat |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Camera | Group or device name define in the see Hierarchical description of the server |
| Description | CDM image pixel format |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | temperatureStatus |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Camera | Group or device name define in the see Hierarchical description of the server |
| Description | Camera temperature status |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | temperatureValue |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Camera | Group or device name define in the see Hierarchical description of the server |
| Description | Camera temperature value |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | Celsius |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | circle_R |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Circle | Group or device name define in the see Hierarchical description of the server |
| Description | LED fitted circle radius vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | circle_RMS |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Circle | Group or device name define in the see Hierarchical description of the server |
| Description | LED fitted circle radius RMS vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | circle_x |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Circle | Group or device name define in the see Hierarchical description of the server |
| Description | LED fitted circle horizontal coo. vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | circle_y |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Circle | Group or device name define in the see Hierarchical description of the server |
| Description | LED fitted circle vertical coo. vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | circle_R_stddev |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Circle | Group or device name define in the see Hierarchical description of the server |
| Description | LED fitted circle radius stddev of vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | circle_RMS_stddev |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Circle | Group or device name define in the see Hierarchical description of the server |
| Description | LED fitted circle radius RMS stddev of vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | circle_x_stddev |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Circle | Group or device name define in the see Hierarchical description of the server |
| Description | LED fitted circle horizontal coo. Stddev of vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | circle_y_stddev |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Circle | Group or device name define in the see Hierarchical description of the server |
| Description | LED fitted circle vertical coo. Stddev of vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | displacement_x |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Displacement | Group or device name define in the see Hierarchical description of the server |
| Description | Displacement between circle center and OARL in horizontal direction vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | displacement_y |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Displacement | Group or device name define in the see Hierarchical description of the server |
| Description | Displacement between circle center and OARL in vertical direction vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | rotation |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.Displacement | Group or device name define in the see Hierarchical description of the server |
| Description | Rotation of the LED circle vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | LED_x_ |  |
| --- | --- | --- |
| Multiplicity | 12 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.LED_x | Group or device name define in the see Hierarchical description of the server |
| Description | LED coo. In horizontal direction vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | LED_y_ |  |
| --- | --- | --- |
| Multiplicity | 12 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.LED_y | Group or device name define in the see Hierarchical description of the server |
| Description | LED coo. In vertical direction vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | OARL_x_ |  |
| --- | --- | --- |
| Multiplicity | 2 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.OARL_x | Group or device name define in the see Hierarchical description of the server |
| Description | OARL coo. In horizontal direction vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | OARL_x_mean |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.OARL_x | Group or device name define in the see Hierarchical description of the server |
| Description | Mean OARL coo. In horizontal direction vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | OARL_y_ |  |
| --- | --- | --- |
| Multiplicity | 2 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.OARL_y | Group or device name define in the see Hierarchical description of the server |
| Description | OARL coo. In vertical direction vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | OARL_y_mean |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.OARL_y | Group or device name define in the see Hierarchical description of the server |
| Description | Mean OARL coo. In vertical direction vector |  |
| Data_Type | double | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | arcsec |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | comment |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | This info will be added to the image header |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | image |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | Compressed image displaying camera image. |  |
| Data_Type | byteString | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | imageName |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | Name of the .fits image |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | imagePath |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | Full .fits image path |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 500 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | imagePath_cat |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | Full .fits image path of all images concatenated. |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | libVersion |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | Version of the CDM library |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | nImagesGet |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | Number of images to take with GetMultipleImages |  |
| Data_Type | int32 | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | numIntegrateImage |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | Number of images to integrate (for taking Tpoints) |  |
| Data_Type | Int32 | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 1 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | status |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | CDM status |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | N/A |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | swVersion |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM | Group or device name define in the see Hierarchical description of the server |
| Description | Software version |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | timestamp_UTC |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.timestamp | Group or device name define in the see Hierarchical description of the server |
| Description | CDM timestamp UTC time vector |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | timestamp_epoch |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.CDM.timestamp | Group or device name define in the see Hierarchical description of the server |
| Description | CDM timestamp Unix time |  |
| Data_Type | string | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 10 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | N/A |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | humidity_absolute |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.Sensor | Group or device name define in the see Hierarchical description of the server |
| Description | Absolute Humidity inside the hood |  |
| Data_Type | float | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | g/m^3 |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | humidity_relative |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.Sensor | Group or device name define in the see Hierarchical description of the server |
| Description | Relative humidity inside the hood |  |
| Data_Type | float | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | % |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | pressure |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.Sensor | Group or device name define in the see Hierarchical description of the server |
| Description | Pressure inside the hood |  |
| Data_Type | float | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | hPa |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

| Name_of_DataPoint | temperature |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone this datapoint |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.Sensor | Group or device name define in the see Hierarchical description of the server |
| Description | Temperature inside the hood |  |
| Data_Type | float | Boolean,Byte,int16,int32,int64,float,double,string,byteString |
| Size | 0 | Size if array (0 if simple type) |
| Default_value | 0 |  |
| Unit | Degrees Celsius |  |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type | Conditional | Not used at this time in the code generator |
| Rate_Condition | In case of Alarm, Error or Warning | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |

#### FSM datapoints (State / Transition)

In this part, we define 2 datapoint you descripbe the statue of the Final State Machine.

The first one describe the status of the state of the FSM.

The second one describe if one transition is called. (Transition between 2 states)

You can see the diagram of the FSM in the next paragraph (Methods of the FSM)

| Name_of_DataPoint | state |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.FSM | Define here the where the datapoint is attached |
| Description | Status state of the FSM |  |
| Data_Type | Int32 | Boolean,Byte,int16,int32,int64,float,double,string |
| Size | 1 | Size if array (0 if simple type) |
| Unit |  | Define here the unit of the datapoint if necessary |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type |  | Not used at this time in the code generator |
| Rate_Condition |  | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Default_value | 1 | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 2 |

Table 6 FSM.state datapoint

| Name_of_DataPoint | transition |  |
| --- | --- | --- |
| Multiplicity | 1 | Define a number if need to clone |
| Start_Number | 1 | If the Multiplicity is greater than 1 define here the start number for the clone |
| Data Exchange |  |  |
| Associate_at | AuxControl.FSM | Define here the where the datapoint is attached |
| Description | status transition of the FSM |  |
| Data_Type | Int32 | Boolean,Byte,int16,int32,int64,float,double,string |
| Size | 1 | Size if array (0 if simple type) |
| Unit |  | Define here the unit of the datapoint if necessary |
| Historization & Subscription |  |  |
| Historization | Y | Not used at this time in the code generator |
| Subscription | Y | Not used at this time in the code generator |
| Data Usage |  |  |
| Engineer |  |  |
| Configuration | Y | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Operator |  |  |
| Configuration | N | Not used at this time in the code generator |
| Control | Y | Not used at this time in the code generator |
| Monitor | Y | Not used at this time in the code generator |
| Data Saving |  |  |
| Type |  | Not used at this time in the code generator |
| Rate_Condition |  | Not used at this time in the code generator |
| Alarms/Errors/Warnings |  |  |
| Type | N/A | Not used at this time in the code generator |
| DP_name | N/A | Not used at this time in the code generator |
| Trigger | N/A | Not used at this time in the code generator |
| Default_value | 1 | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| Action | N/A | Not used at this time in the code generator |
| GUI_Expert | 2 | 2 |

Table 7 FSM.transition datapoint

### Description of configuration Datapoints

This chapter describe in detail all the data points defined as a control value limit for the CDM CCD.

### Description of sub servers

This part allow to describe the reference of sub OpcUa servers.

This reference allow concentrating the address/port definition in one place.

When some methods describe bellow need to make a gateway with other sub Servers, you need to define the name server that you want to connect.

- You define a reference in the table below with the real OpcUa address of the Server.

- You define, in the good “method table” in the chapter “description of methods”, the tag <RemoteServerId> with the reference define bellow.

| serverA | opc.tcp://CDMserver:48011 |
| --- | --- |

Table 8 SubServer reference

### Description of sub elements

This part allow describing the reference (Datapoints or methods) needed for making the bridge.

This reference allow concentrating the nameId definition in one place.

When some methods describe below need to make a gateway with other sub Servers, you need to define the name server but also the NodeId that you want to connect and associate

- You define a reference in the table below with the real OpcUa nodeId.

- You define, in the good “method table” in the chapter “description of methods”, the tag <RemoteNodeId> with the reference define bellow.

2 options (Method or Datapoint to access)

Access to one method in the sub server

  - Define the nodeId without the name of the method in the table below.

  - You define, in the good “method table” in the chapter “description of methods”, the tag <NodeId> with the reference define bellow.

  - You define, in the good “method table” in the chapter “description of methods”, the tag <MethodId> with the name of the method you want to access.

Access to one Datapoint in the sub Server

  - Define completely the nodeId in this table below.

  - You define, in the good “method table” in the chapter “description of methods”, the tag <NodeId> with the reference define bellow.

  - You erase , in the good “method table” in the chapter “description of methods”, the tag <MethodId>

Table 9: SubDP reference

| Unit_RemotePowerCtrl |  |
| --- | --- |
| Ref1 | Unit_CDM.AuxControl.FSM |

### Description of Methods

| Name_of_Method | Connect |  |
| --- | --- | --- |
| Description | Connect to the camera |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | Connect |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | Disconnect |  |
| --- | --- | --- |
| Description | Disconnect to the camera |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | Disconnect |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | Configure |  |
| --- | --- | --- |
| Description | Configures the camera settings |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | in seconds |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Input parameters |  |  |
| Name | Input_pixel_clock |  |
| Data_Type | int32 | Boolean,int8,int16,int32,int64,float,double,string |
| Size | 1 | Size if array (0 if simple type) |
| Type_Size_for_ACTL | TBD |  |
| Description | Desired sensor refresh rate |  |
| Min_Range | 96 |  |
| Max_Range | 432 |  |
| Unit | MHz |  |
| Input parameters |  |  |
| Name | Input_exposure |  |
| Data_Type | double | Boolean,int8,int16,int32,int64,float,double,string |
| Size | 1 | Size if array (0 if simple type) |
| Type_Size_for_ACTL | TBD |  |
| Description | Desired camera exposure |  |
| Min_Range | 0.01 |  |
| Max_Range | 1000 |  |
| Unit | ms |  |
| Input parameters |  |  |
| Name | Input_FPS |  |
| Data_Type | double | Boolean,int8,int16,int32,int64,float,double,string |
| Size | 1 | Size if array (0 if simple type) |
| Type_Size_for_ACTL | TBD |  |
| Description | Desired camera frames per second |  |
| Min_Range | 1 |  |
| Max_Range | 20 |  |
| Unit | Hz |  |
| Input parameters |  |  |
| Name | Input_gain |  |
| Data_Type | int32 | Boolean,int8,int16,int32,int64,float,double,string |
| Size | 1 | Size if array (0 if simple type) |
| Type_Size_for_ACTL | TBD |  |
| Description | Desired camera gain |  |
| Min_Range | 0 |  |
| Max_Range | 100 |  |
| Unit | N/A |  |
| Input parameters |  |  |
| Name | Input_pixel_format |  |
| Data_Type | string | Boolean,int8,int16,int32,int64,float,double,string |
| Size | 1 | Size if array (0 if simple type) |
| Type_Size_for_ACTL | TBD |  |
| Description | Desired pixel format. "IS_CM_SENSOR_RAW16" or "IS_CM_MONO8" |  |
| Min_Range |  |  |
| Max_Range |  |  |
| Unit | N/A |  |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | Just for generate a generic GUI in python. (Beta test) |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | Configure |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | AddComment |  |
| --- | --- | --- |
| Description | Add a comment to be saved |  |
| Associate_at | AuxControl.CDM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Input parameters |  |  |
| Name | Input_comment |  |
| Data_Type | string | Boolean,int8,int16,int32,int64,float,double,string |
| Size | 1 | Size if array (0 if simple type) |
| Description | This will be added to the image header. |  |
| Min_Range |  |  |
| Max_Range |  |  |
| Unit | N/A |  |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | false |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | AddComment |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | GetImage |  |
| --- | --- | --- |
| Description | Take an image |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | GetImage |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | StartCDM |  |
| --- | --- | --- |
| Description | Start continuous image acqusition |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 10 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Asynchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | StartCDM |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | StopCDM |  |
| --- | --- | --- |
| Description | Stop continuous image acqusition |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | StopCDM |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method |  | StartStream |  |  |
| --- | --- | --- | --- | --- |
| Description |  | Start camera video stream |  |  |
| Associate_at |  | AuxControl.CDM.FSM |  |  |
| Return_Type |  | Int32 |  | OPC_UA status |
| Method_Timeout |  | 10 |  | In seconds. Not used at this time in the code generator |
| A_Synchronous |  | Asynchronous |  | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |  |  |
| Type |  | TBD |  | Not used at this time in the code generator |
| Code |  | TBD |  | Not used at this time in the code generator |
| Trigger |  | TBD |  | Not used at this time in the code generator |
| Action |  | TBD |  | Not used at this time in the code generator |
| GUI_Expert |  | 2 |  | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM |  | true |  |  |
| MOS_Information |  |  |  |  |
| RemoteServer |  | serverA |  |  |
| RemoteMethodPath |  | Ref1 |  |  |
| RemoteMethodName |  | StartStream |  |  |
| RemoteNamespace |  | 2 |  |  |
| PluginDeviceInstruction |  |  |  |  |
| Name_of_Method | StopStream |  |  |  |
| Description | Stop camera video stream |  |  |  |
| Associate_at | AuxControl.CDM.FSM |  |  |  |
| Return_Type | Int32 |  | OPC_UA status |  |
| Method_Timeout | 2 |  | In seconds. Not used at this time in the code generator |  |
| A_Synchronous | Synchronous |  | Synchronous/Asynchronous |  |
| Alarms/Errors/Warnings |  |  |  |  |
| Type | TBD |  | Not used at this time in the code generator |  |
| Code | TBD |  | Not used at this time in the code generator |  |
| Trigger | TBD |  | Not used at this time in the code generator |  |
| Action | TBD |  | Not used at this time in the code generator |  |
| GUI_Expert | 2 |  | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |  |
| FSM | true |  |  |  |
| MOS_Information |  |  |  |  |
| RemoteServer | serverA |  |  |  |
| RemoteMethodPath | Ref1 |  |  |  |
| RemoteMethodName | StopStream |  |  |  |
| RemoteNamespace | 2 |  |  |  |
| PluginDeviceInstruction |  |  |  |  |

| Name_of_Method | Error |  |
| --- | --- | --- |
| Description | If an error occurs |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | Error |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | Acknowledge |  |
| --- | --- | --- |
| Description | Acknowledge error |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | Acknowledge |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | GoToTpoint |  |
| --- | --- | --- |
| Description | Go to Tpoint state |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | GoToTpoint |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | GoToReady |  |
| --- | --- | --- |
| Description | Return to ready state from Tpoint state |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | GoToReady |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | GetMultipleImages |  |
| --- | --- | --- |
| Description | Takes multiple images |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 10 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Asynchronous | Synchronous/Asynchronous |
| Input parameters |  |  |
| Name | N_images |  |
| Data_Type | int32 |  |
| Size | 1 | Size if array (0 if simple type) |
| Description | Number of images to take | Size if array (0 if simple type) |
| Min_Range | 1 |  |
| Max_Range | 500 |  |
| Unit | N/A |  |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | GetMultipleImages |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | StopGetMultipleImages |  |
| --- | --- | --- |
| Description | Stop taking Tpoint images |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 2 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Synchronous | Synchronous/Asynchronous |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | StopGetMultipleImages |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |

| Name_of_Method | GetMultipleImagesStacked |  |
| --- | --- | --- |
| Description | Takes multiple images, averages them and saves as one image |  |
| Associate_at | AuxControl.CDM.FSM |  |
| Return_Type | Int32 | OPC_UA status |
| Method_Timeout | 10 | In seconds. Not used at this time in the code generator |
| A_Synchronous | Asynchronous | Synchronous/Asynchronous |
| Input parameters |  |  |
| Name | N_images |  |
| Data_Type | int32 |  |
| Size | 1 | Size if array (0 if simple type) |
| Description | Number of images to take | Size if array (0 if simple type) |
| Min_Range | 1 |  |
| Max_Range | 500 |  |
| Unit | N/A |  |
| Alarms/Errors/Warnings |  |  |
| Type | TBD | Not used at this time in the code generator |
| Code | TBD | Not used at this time in the code generator |
| Trigger | TBD | Not used at this time in the code generator |
| Action | TBD | Not used at this time in the code generator |
| GUI_Expert | 2 | 1 = operator , 2 = semi expect, 3 =database, 4 = expert |
| FSM | true |  |
| MOS_Information |  |  |
| RemoteServer | serverA |  |
| RemoteMethodPath | Ref1 |  |
| RemoteMethodName | GetMultipleImagesStacked |  |
| RemoteNamespace | 2 |  |
| PluginDeviceInstruction |  |  |